PHYSICAL REVIEW B 90, 064111 (2014)

Pathways towards ferroelectricity in hafnia

Tran Doan Huan, Vinit Sharma, George A. Rossetti, Jr., and Rampi Ramprasad* 
Department of Materials Science & Engineering and Institute of Materials Science, University of Connecticut, 97 North Eagleville Rd., Unit 3136, Storrs, Connecticut 06269-3136, USA 
(Received 5 July 2014; revised manuscript received 28 July 2014; published 27 August 2014)

The question of whether one can systematically identify (previously unknown) ferroelectric phases of a given material is addressed, taking hafnia (HfO2) as an example. Low free energy phases at various pressures and temperatures are identified using a first-principles based structure search algorithm. Ferroelectric phases are then recognized by exploiting group theoretical principles for the symmetry-allowed displacive transitions between nonpolar and polar phases. Two orthorhombic polar phases occurring in space groups Pca21 and Pmn21 are singled out as the most viable ferroelectric phases of hafnia, as they display low free energies (relative to known nonpolar phases), and substantial switchable spontaneous electric polarization. These results provide an explanation for the recently observed surprising ferroelectric behavior of hafnia, and reveal pathways for stabilizing ferroelectric phases of hafnia as well as other compounds.

DOI: 10.1103/PhysRevB.90.064111

Commonly known structural phases of hafnia (HfO2) are centrosymmetric, and thus nonpolar. Hence recent observa-tions of ferroelectric behavior of hafnia thin films (when doped with Si, Zr, Y, Al, or Gd) [1–7] are rather surprising as ferroelectricity requires the presence of switchable spon-taneous electrical polarization. The emergence of nonpolar hafnia—as a linear high dielectric constant (or high-κ) successor to SiO2—for use in modern electronic devices (e.g., field-effect transistors) is now well established [8,9]. If the origins of this unexpected ferroelectricity can be understood and appropriately harnessed, hafnia-based materials may find applications in nonvolatile memories and ferroelectric field

PACS number(s): 77.84.−s, 77.65.−j, 81.05.Zx

These results are consistent with available experimental data [8,9,14–19]. Figure 1(b) shows the temperature and pressure regimes [overlaid on the phase diagram of Fig. 1(a)] when the identified ferroelectric Pca21 and Pmn21 orthorhombic phases are extremely close in free energy (<kBT /5, where kB is the Boltzmann constant) to the equilibrium nonpolar phases. The computed spontaneous polarizations of these ferroelectric phases are substantial, and switchable (at low temperatures) with small energy barriers (∼10 meV/atom) via the parent nonpolar P42/nmc tetragonal phase. Thus Pca21 and Pmn21 are viable ferroelectric phases and may be stabilized under appropriate experimental conditions (e.g., via strain due to

effect transistors as well. 	epitaxy or dopants).

A broader question that arises within this context, and also the one that will be addressed directly in this contribution, is whether one can systematically identify ferroelectric phases of a given material system. We show that this can indeed be accomplished and ascertained, for the example of hafnia, in two steps. First, a computation-based structure search method, e.g., the minima-hopping method [10–12], is used to identify low-energy phases at various pressures and temperatures. Then, ferroelectric phases are singled out by applying the group theoretical symmetry reduction principles, established by Shuvalov for ferroelectricity [13]. These principles al-low for the systematic identification of all possible lower symmetry proper ferroelectric phases that can result from

Our density functional theory (DFT) [20,21] calculations were performed with the ABINIT package [22,23], employ-ing the norm-conserving Hartwigsen-Goedecker-Hutter pseu-dopotentials [24] and the Perdew-Burke-Ernzerhof exchange-correlation energies functional [25]. Highly accurate DFT energies EDFT were ensured by dense Monkhorst-Pack k-point meshes [26], and a basis set of plane waves with kinetic energy up to 1100 eV. Convergence is assumed when the residual forces and stresses are smaller than 10−3eV/˚A and 10−5eV/˚A3, respectively. As given in the Supplemental Material [27], the lattice parameters obtained by optimizing for the previously known phases of hafnia agree very well with experimental data, implying that our computational scheme is

higher-symmetry nonpolar prototype (parent) phases. 	reasonable.

Using this approach, we find two ferroelectric phases of hafnia, belonging to the Pca21 and Pmn21 orthorhom-bic space groups, which are close in free energy with the known nonpolar equilibrium phases of hafnia over a wide temperature and pressure range. Figure 1(a) displays the computed equilibrium phase diagram of hafnia indicating the regimes at which the known nonpolar phases are stable. This includes the low-temperature low-pressure P21/c monoclinic phase, high-pressure Pbca and Pnma orthorhombic phases, and the high-temperature P42/nmc tetragonal phase (the high-temperature Fm3m cubic aristotype is not shown).

Given that the ferroelectricity was observed [1–7] in doped hafnia thin films, and that pressure is theoretically predicted [28] to play an important role in stabilizing the Pca21 polar phase, we systematically searched for low-energy structures at pressures up to 30 GPa. In the minima-hopping method [10–12], used for our search, the DFT energy landscape of hafnia is explored by performing consecutive short molecular-dynamics steps, traveling across local minima, progressing towards the global minimum. The initial velocities of the molecular-dynamics trajectories were chosen to lay approximately along soft mode directions, improving the efficiency of the method. Local geometry optimizations were

then used to obtain the equilibrium structure. This method

*rampi@ims.uconn.edu 	has been successfully applied in the past for various classes

1098-0121/2014/90(6)/064111(5) 	064111-1	©2014 American Physical Society

| HUAN, SHARMA, ROSSETTI JR., AND RAMPRASAD |  |  |  |  |  |  |  |  |  |  |  |  | F (meV/atom) | 60 | 0K | PHYSICAL REVIEW B 90, 064111 (2014) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.5 |  | Tetragonal (P42/nmc) |  |  | (a) |  |  | Orthorhombic (Pmn21) |  |  | (b) |  |  |  |  | P21/c P42/nmc Pbca Pnma | Pca21 Pmn21 P21/m |
| 2.0 |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  |  |  |
| T (1000 K) | 1.5 | Monoclinic (P21/c) |  | Orthor-hombic (Pbca) | Orthor- hombic (Pnma) |  |  |  | Orthorhombic (Pca21) |  |  |  |  | 20 |  | 300K |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |  |
|  | 1.0 |  |  |  |  |  |  |  |  |  |  |  |  | -20 |  |  |  |
| 0.5 |  |  |  |  |  |  |  |  |  |  |  |  | F (meV/atom) | 60 | 1800K | 2200K |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  |  |  |
| 0.0 |  | 0 | 10 |  | 20 | 30 | 0 |  |  | 10 | 20 | 30 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |
| P (GPa) |  |  |  |  | P (GPa) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |  |
| FIG. 1. (Color online) (a) Computed equilibrium phase diagram |  |  |  |  |  |  |  |  |  |  |  |  |  | -20 |  |  |  |

of hafnia and (b) regimes in which �F(P,T ), the free energy difference between Pca21 and Pmn21 phases, and the equilibrium phases are small, i.e., �F(P,T ) < kBT/5.

of crystalline materials, including inorganic [29–31] and organometallic [32] compounds.

The structure search procedure led to a large number of possible hafnia phases. The eleven lowest-energy cases included six nonpolar and five polar phases. The six non-polar phases were the previously known P21/c monoclinic, P42/nmc tetragonal, Pbca and Pnma orthorhombic, and Fm3m cubic phases, as well as the recently predicted P21/m monoclinic phase [33]; the five polar phases were the Pca21 and Pmn21 orthorhombic, Pm and Cc monoclinic, and the P1 triclinic phases. Phonon band structure calculations, performed at zero temperature and various finite pressures up to 30 GPa using the linear response method [34,35], reveal that with the two exceptions of Fm3m and P21/m, the remaining structures are dynamically stable across the entire pressure range. The high-temperature Fm3m cubic structure is dynamically unstable within the whole pressure range exam-ined, and the P21/m structure is dynamically stable only below 15 GPa. Above this pressure, the P21/m structure collapses to the P42/nmc tetragonal structure (as ascertained by following the unstable phonon modes). More detailed crys-tallographic information, simulated x-ray diffraction patterns, and phonon frequency spectra of the examined structures are given in the Supplemental Material [27].

From the calculated phonon band structures, free energies F were computed within the harmonic approximation for the dynamically stable structures. These are shown in Fig. 2 for four different temperatures across the entire pressure range considered. As established previously [15,16,18,19], we found that the P21/c, Pcba, and Pnma phases are thermodynamically stable below 10 GPa, between 10 GPa and 20 GPa, and above 20 GPa, respectively. Consistent with a recent experimental report [36], our additional calculations reveal that up to 100 GPa, the Pnma structure remains the most stable phase of hafnia. The stabilization of the P42/nmc tetragonal phase is also confirmed in our work. Starting from 1800 K and 10 GPa, this phase is stable relative to the P21/c monoclinic and the Pcba orthorhombic phases. At ambient pressures, we predict that hafnia transforms from the P21/c experiments [18,37]. The phase diagram 1(a), which was phase to the P42/nmc phase at Tc ≃ 2200 K, consistent with

0 	10 	20 	30 	0 	10 	20 	30 	P (GPa) 	P (GPa)

FIG. 2. (Color online) Free energies F calculated at T = 0 K, T = 300 K, T = 1800 K, and T = 2200 K for the identified low-energy phases of hafnia and shown as functions of pressure P. Data is given by symbols while curves are guides to the eye. Data corresponding to the Pm, Cc, and P1 phases are not shown as they are energetically unfavorable by roughly 250 meV/atom compared with the other phases.

Next, we consider the predicted non-centrosymmetric hafnia phases, i.e., phases whose point groups do not contain an inversion operation. We note that of the five cases identified, the two orthorhombic phases Pca21 and Pmn21 are energetically competing with the equilibrium phases, while the Pm, Cc, and P1 phases are unfavorable by roughly 250 meV/atom compared to the other phases. Figure 1(b) identifies the temperature and pressure regimes at which the Pca21 and Pmn21 phases are extremely close in free energy (<kBT /5) to the corresponding equilibrium phases. Indeed, the Pca21 orthorhombic phase has been observed in zirconia doped with Mg, Ca, and Y [38–41] and recently suggested [28,42] to be a low-energy polar phase of both hafnia and zirconia. The other four phases, i.e., Pmn21, Pm, Cc, and P1, are identified in this work.

According to Shuvalov [13], the point symmetry mm2 displayed by Pca21 and Pmn21 phases, the point sym-metry m displayed by Pm and Cc phases, and the point symmetry 1 displayed by P1 phase, are three ferroelectric subgroups of the 4/mmm tetragonal point group. In fact, the symmetry requirements placed on ferroelectric phases that can result from a parent nonpolar phase through distortions are quite definitive. For instance, starting from a tetragonal prototype phase with point symmetry 4/mmm, ferroelectric orthorhombic phases of only two types with point symmetry mm2 (each of them accompanied by a definite polar axis) are possible. These principles suggest that the P42/nmc tetragonal phase of hafnia is the parent nonpolar phase for all identified polar phases. Indeed, we found that all five non-

centrosymmetric structures considered herein can be obtained

by distorting the P42/nmc tetragonal structure appropriately. Two most interesting cases, the Pca21 and Pmn21 orthorhom-bic structures, can be obtained by distorting the P42/nmc

tetragonal structure along its [110] and [100] directions,

constructed from the calculated F, summarizes these findings. 	respectively.

064111-2

PATHWAYS TOWARDS FERROELECTRICITY IN HAFNIA

FIG. 3. (Color online) Symmetry-reduction flowchart of low-energy phases of hafnia, starting from the m3m cubic point symmetry. These phases, labeled by space group symbols, are categorized into two branches, the nonpolar branch ending at the 2/m point symmetry on the left and the polar branch leading to the 1 point symmetry on the right. Dark yellow and red spheres represent hafnium and oxygen atoms, respectively, while green spheres locate the center of symmetry of those which are centrosymmetric.

In order to better appreciate our findings within the context of Shuvalov’s symmetry-reduction principles, we show in Fig. 3 all the predicted low-energy phases of hafnia identified by our structure-search scheme, classified in terms of their symmetry. The right branch of Fig. 3 shows the five identified polar phases (grouped in terms of their point symmetries)

PHYSICAL REVIEW B 90, 064111 (2014)

emerging from the prototype P42/nmc tetragonal phase. For completeness, we also display the nonpolar phases on the left branch of Fig. 3. The highest symmetry phase at the top of the figure is the Fm3m cubic phase. Successive symmetry reduction leads to the P42/nmc tetragonal phase, the high-pressure Pcba and Pnma orthorhombic phases, and, finally, the ground-state P21/c monoclinic phase and the recently predicted P21/m monoclinic phase [33].

We now consider just the polar Pca21 and Pmn21 or-thorhombic phases, and investigate the magnitude and switch-ability of the polarization. Taking the P42/nmc tetragonal phase as the reference, the spontaneous polarization was computed (through the evaluation of the Berry phase at T = 0 K [43,44]) to be PPca21 = 52 μC/cm2and PPmn21 = 56 μC/cm2pointing along the [001] and [100] directions, re-spectively (the [001] direction of the Pca21 phase corresponds to the [110] direction of the prototype P42/nmc phase). Our calculated spontaneous polarization is in good agreement with experimentally measured values of remanent and saturation polarization (which fall within 20–50 μC/cm2[1–7]), as well as with recent computations for the Pca21 phase of hafnia [28,42].

The spontaneous polarization of the Pca21 and Pmn21 phases are switchable with small energy barriers for 180◦switching. For each of these phases, there are two topologically

equivalent variants with opposite polarization, referred to as

“up” and “down” states. These variants can be obtained by de-forming the prototype P42/nmc structure along two opposite directions (see Fig. 4 for an illustration). We parametrized the continuous deformation from the up to the down states by λ, a parameter scaled to range from −1 to 1. The minimum (DFT) energy pathways describing the deformation leading to the Pca21 and Pmn21 phases, calculated using the generalized solid-state nudged elastic band method [45], are shown in Fig. 4. As expected, these pathways go through the P42/nmc tetragonal phase at their centers (λ = 0) before reaching the opposite polarization state. The curvatures of the calculated pathways at λ = 0 are very small, indicating that although the tetragonal phase is dynamically stable, it corresponds to a very shallow minimum of the energy landscape (the corresponding phonon soft modes can be observed in the Supplemental Material [27]). Experimentally, the transition from P42/nmc

FIG. 4. (Color online) Minimum (DFT) energy pathways from the “up” state (λ = −1) to the “down” state (λ = 1) of two polar phases,

Pca21 on the left panel and Pmn21 on the right panel. Illustrations for the structural arrangements along the pathways are given, using dark

yellow and red spheres for hafnium and oxygen atoms, respectively. Long blue arrows indicate the deformation directions along the pathways,

while short red arrows represent the polarizations of the proposed polar phases. The polarization of the reference phase (P42/nmc) is zero.

064111-3

| HUAN, SHARMA, ROSSETTI JR., AND RAMPRASAD Monoclinic (P21/c) |  |  | PHYSICAL REVIEW B 90, 064111 (2014) suggests that while en route to the polar phases (Pca21 and Pmn21) of hafnia, the P42/nmc tetragonal phase has to be stabilized. We note in particular that the P42/nmc phase may be stabilized by some suitable combination of stress and/or internal/external electric field, e.g., by chemical doping or by fabrication of strain-engineered thin film structures [9,46]. In Fig. 5 we tentatively sketch possible pathways which may enable the formation of polar phases even in pure hafnia. These pathways include several processes. First, the P42/nmc tetragonal phase may be stabilized under suitable conditions, e.g., T ≃ 2200 K at ambient pressure or T ≃ 1800 K at P ≃ 10 GPa. Next, perturbations, e.g., via an electric field, could drive the P42/nmc phase to a ferroelectric phase, either Pca21 or Pmn21. Finally, samples may be quenched to lower temperatures and pressures, preserving the obtained polar phases. In summary, we have systematically identified possible ferroelectric phases of hafnia from first principles by com- |
| --- | --- | --- | --- |
| Stabilizing | T > 2200 K | T > 1800 K |  |
|  |  | P > 10 GPa |  |
| Tetragonal (P42/nmc) Polarizing |  |  |  |
| Orthorhombic |  | Orthorhombic |  |
| (Pca21) |  | (Pmn21) |  |
| Quenching |  |  |  |
| Orthorhombic |  | Orthorhombic |  |
| (Pca21) |  | (Pmn21) |  |

FIG. 5. (Color online) Possible pathways which might describe

the formation process of hafnia polar phases. First, the P42/nmc

tetragonal phase is stabilized at suitable temperature and/or pressure.

Next, this phase may be driven into a polar phase in the subsequent

polarizing step. Finally, the polar phase may be quenched to lower

temperatures and pressures.

to Pca21, hypothesized to be a polar phase of hafnia, was

conjectured in Ref. [6].

Moreover, the transitions from the P42/nmc phase to the

Pca21 and Pmn21 phases are also allowed by thermodynamic

considerations. In particular, the energy barriers separating the

up and the down states of the Pca21 and Pmn21 phases at

P = 0 8 meV/atom, respectively. Between the P42/nmc phase 	GPa 	are 	estimated 	to 	be 	40 	meV/atom 	and

and these two ferroelectric phases, the energy barriers are

smaller, i.e., 6 meV/atom for the Pca21 phase and less than

1 meV/atom for the Pmn21 phase. For zirconia, the structural

phase transition from P42/nmc to Pca21 under hydrostatic

pressures has experimentally been observed [40].

The present findings provide a rationale for the observed

bining a low-energy structure prediction method and sym-metry reduction principles established for ferroelectric phase transitions. Two of the identified orthorhombic polar phases, which belong to the Pca21 and Pmn21 space groups, are

found to be extremely close in free energy to the equilibrium

nonpolar phases over a wide temperature and pressure window.

Both polar phases can be obtained by distorting the well-

known prototype nonpolar P42/nmc tetragonal structure of hafnia. The calculated spontaneous polarization is substantial and switchable (with low-energy barriers), implying that the Pca21 and Pmn21 phases may provide an explanation for the recent observation of the ferroelectricity in doped hafnia films. The scheme presented here can, in principle, be used to systematically identify pathways towards ferroelectricity in other compounds as well.

The authors thank Jacob Jones for drawing their attention to this problem and for subsequent stimulating discussions. The authors also thank S. Goedecker and M. Amsler for making the minima-hopping code available. The x-ray diffrac-tion patterns of the examined phases were simulated by FULLPROF [47], while their space groups were determined by FINDSYM [48]. Some figures in this work were rendered with

| ferroelectricity in hafnia under suitable conditions. Figure 4 | VESTA [49]. |
| --- | --- |

[1] T. S. B¨oscke, J. M¨uller, D. Br¨auhaus, U. Schr¨oder, and 	U. B¨ottger, Appl. Phys. Lett. 99, 102903 (2011).

[2] T. S. B¨oscke, S. Teichert, D. Br¨auhaus, J. M¨uller, U. Schr¨oder,

[6] T. S. B¨oscke, J. M¨uller, D. Br¨auhaus, U. Schr¨oder, and U. B¨ottger, in Proceedings of the 2011 IEEE International Elec-tron Devices Meeting (IEDM) (IEEE, Washington, DC, 2011),

U. B¨ottger, and T. Mikolajick, Appl. Phys. Lett. 99, 112904 	pp. 24.5.1–24.5.4.

(2011). 	[7] P. D. Lomenzo, P. Zhao, Q. Takmeel, S. Moghaddam, T. Nishida,

[3] S. Mueller, J. Mueller, A. Singh, S. Riedel, J. Sundqvist, U. Schroeder, and T. Mikolajick, Adv. Funct. Mater. 22, 2412

M. Nelson, C. M. Fancher, E. D. Grimley, X. Sang, J. M. LeBeau, and J. L. Jones, J. Vac. Sci. Technol. B 32, 03D123 (2014).

(2012). 	[8] G. D. Wilk, R. M. Wallace, and J. M. Anthony, J. Appl. Phys.

[4] J. M¨uller, T. S. B¨oscke, U. Schr¨oder, S. Mueller, D. Br¨auhaus, 	87, 484 (2000).

U. B¨ottger, L. Frey, and T. Mikolajick, Nano Lett. 12, 4318 	[9] H. Zhu, C. Tang, L. Fonseca, and R. Ramprasad, J. Mater. Sci.

(2012). 	47, 7399 (2012).

[5] S. Mueller, C. Adelmann, A. Singh, A. Van Elshocht, U. Schroeder, and T. Mikolajick, ECS J. Solid State Sci. Technol.

[10] S. Goedecker, J. Chem. Phys. 120, 9911 (2004).

[11] M. Amsler and S. Goedecker, J. Chem. Phys. 133, 224104

1, N123 (2012). 	(2010).

064111-4

PATHWAYS TOWARDS FERROELECTRICITY IN HAFNIA

[12] S. Goedecker, in Modern Methods of Crystal Structure Predic-

tion, edited by A. R. Oganov (Wiley-VCH, Weinheim, 2011),

PHYSICAL REVIEW B 90, 064111 (2014)

[28] S. Clima, D. J. Wouters, C. Adelmann, T. Schenk, U. Schroeder,

M. Jurczak, and G. Pourtois, Appl. Phys. Lett. 104, 092906

pp. 147–180, Chap. 7. 	(2014).

[13] L. A. Shuvalov, J. Phys. Soc. Jpn. (Suppl.) 28, 38 (1970). [14] R. Ruh and P. W. R. Corfield, J. Am. Ceram. Soc. 53, 126 (1970).

[29] M. Amsler, J. A. Flores-Livas, T. D. Huan, S. Botti, M. A. L. Marques, and S. Goedecker, Phys. Rev. Lett. 108, 205505

[15] L.-G. Liu, J. Phys. Chem. Solids 41, 331 (1980). 	(2012).

[16] J. M. Leger, A. Atouf, P. E. Tomaszewski, and A. S. Pereira, 	Phys. Rev. B 48, 93 (1993).

[17] O. Ohtaka, T. Yamanaka, and S. Kume, Nippon Seramikkusu Kyokai Gakujutsu Ronbunshi (J. Ceram. Soc. Jpn.) 99, 826 (1991).

[18] O. Ohtaka, H. Fukui, T. Kunisada, T. Fujisawa, K. Funakoshi, W.

Utsumi, T. Irifune, K. Kuroda, and T. Kikegawa, J. Am. Ceram. Soc. 84, 1369 (2001).

[19] J. Tang, M. Kai, Y. Kobayashi, S. Endo, O. Shimomura, T. Kikegawa, and T. Ashida, in Properties of Earth and Planetary Materials at High Pressure and Temperature, edited by M. H. Manghnani and T. Yagi (American Geophysical Union, Washington, DC, 2013), pp. 401–407.

[20] P. Hohenberg and W. Kohn, Phys. Rev. 136, B864 (1964).

[30] T. D. Huan, M. Amsler, M. A. L. Marques, S. Botti, A. Willand, 	and S. Goedecker, Phys. Rev. Lett. 110, 135502 (2013).

[31] T. D. Huan, M. Amsler, R. Sabatini, V. N. Tuoc, N. B. Le, L. M.

Woods, N. Marzari, and S. Goedecker, Phys. Rev. B 88, 024108 (2013).

[32] A. F. Baldwin, R. Ma, A. Mannodi-Kanakkithodi, T. D. Huan, C. Wang, M. Tefferi, J. E. Marszalek, M. Cakmak, Y. Cao, R. Ramprasad, and G. A. Sotzing (unpublished).

[33] Q. Zeng, A. R. Oganov, A. O. Lyakhov, C. Xie, X. Zhang, J.

Zhang, Q. Zhu, B. Wei, I. Grigorenko, L. Zhang, and L. Cheng, Acta Crystallogr. Sect. C 70, 76 (2014).

[34] C. Lee and X. Gonze, Phys. Rev. B 51, 8610 (1995).

[35] X. Gonze and C. Lee, Phys. Rev. B 55, 10355 (1997).

[36] Y. Al-Khatatbeh, K. K. M. Lee, and B. Kiefer, Phys. Rev. B 82,

| [21] W. Kohn and L. Sham, Phys. Rev. 140, A1133 (1965). |  |  |  |  |  |  | Beuken, | 144106 (2010). |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [22] X. | Gonze, | B. | Amadon, | P.-M. | Anglade, | J.-M. |  | [37] J. Wang, H. Li, and R. Stevens, J. Mater. Sci. 27, 5397 (1992). |

F. Bottin, P. Boulanger, F. Bruneval, D. Caliste, R. Caracas, M. Cˆot´e, T. Deutsch, L. Genovese, P. Ghosez, M. Giantomassi, S. Goedecker, D. Hamann, P. Hermet, F. Jollet, G. Jomard, S. Leroux, M. Mancini, S. Mazevet, M. Oliveira, G. Onida, Y. Pouillon, T. Rangel, G.-M. Rignanese, D. Sangalli, R.

Shaltaf, M. Torrent, M. Verstraete, G. Zerah, and J. Zwanziger, Comput. Phys. Commun. 180, 2582 (2009).

[23] X. Gonze, G. M. Rignanese, M. Verstraete, J.-M. Beuken, Y.

[38] A. Heuer, V. Lanteri, S. Farmer, R. Chaim, R. Lee, B. Kibbel, 	and R. Dickerson, J. Mater. Sci. 24, 124 (1989).

[39] D. B. Marshall, M. R. Jarnes, and J. R. Porter, J. Am. Ceram. 	Soc. 72, 218 (1989).

[40] E. H. Kisi, J. Am. Ceram. Soc. 81, 741 (1998).

[41] C. J. Howard, E. H. Kisi, R. B. Roberts, and R. J. Hill, J. Am. 	Ceram. Soc. 73, 2828 (1990).

| [42] S. | E. | Reyes-Lillo, | K. | F. | Garrity, | and | K. | M. | Rabe, |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Pouillon, R. Caracas, F. Jollet, M. Torrent, G. Zerah, M. Mikami, 	arXiv:1403.3878.

P. Ghosez, M. Veithen, J.-Y. Raty, V. Olevano, F. Bruneval, L. 	[43] R. D. King-Smith and D. Vanderbilt, Phys. Rev. B 47, 1651 Reining, R. Godby, G. Onida, D. R. Hamann, and D. C. Allan, 	(1993).

Z. Kristallogr. 220, 558 (2005).

[24] C. Hartwigsen, S. Goedecker, and J. Hutter, Phys. Rev. B 58,

[44] R. Resta, Rev. Mod. Phys. 66, 899 (1994).

[45] D. Sheppard, P. Xiao, W. Chemelewski, D. D. Johnson, and

3641 (1998). 	G. Henkelman, J. Chem. Phys. 136, 074103 (2012).

[25] J. P. Perdew, K. Burke, and M. Ernzerhof, Phys. Rev. Lett. 77, 	[46] L. G. Wang, Y. Xiong, W. Xiao, L. Cheng, J. Du, H. Tu, and

3865 (1996). 	A. van de Walle, Appl. Phys. Lett. 104, 201903 (2014).

[26] H. J. Monkhorst and J. D. Pack, Phys. Rev. B 13, 5188 (1976). [27] See Supplemental Material at http://link.aps.org/supplemental/ 	10.1103/PhysRevB.90.064111 for more information reported in 	this paper.

[47] J. Rodr´ıguez-Carvajal, Physica B 192, 55 (1993).

[48] FINDSYM, http://stokes.byu.edu/findsym.html.

[49] K. Momma and F. Izumi, J. Appl. Crystallogr. 41, 653 	(2008).

064111-5